RIGHT OF WAY AND CENTERLINE MONUMENTS 	(note on the Centerline Plat, rev 10-1-25)

SETTING OF ALL MONUMENTS SHALL BE PERFORMED BY OR UNDER THE DIRECT SUPERVISION OF A SURVEYOR REGISTERED IN THE STATE OF OHIO.

monumentation to be set will be in accordance with ODOT standard drawing RM-1.1, Type “C” monuments, AS WELL AS 8” mag spikes, AS CALLED FOR IN THE PLANS.

THE CONTRACTOR’S RESPONSIBILITIES AND Pay ITEMS for INSTALLING MONUMENT ASSEMBLIES AND COORDINATING WITH THE DESIGN CONSULTANT’S SURVEYOR ARE shown in the general notes and general summary under coc item 623 – MONUMENT ASSEMBLY, TYPE C.

THE MONUMENT ASSEMBLIES AND REFERENCE MONUMENT ASSEMBLIES WILL BE INSTALLED BY THE CONTRACTOR.  THE IRON PIN AND CAP WILL BE INSTALLED BY THE DESIGN CONSULTANT’S SURVEYOR OF RECORD.

It is the RESPONSIBILITY OF THE DESIGN consultant’s SURVEYOR to set the property corners as defined by Ohio Administrative Code 4733-37.

Changes or alterations to the location of any monuments shown on this table requires prior approval from the Public Service, Design and Construction, RIGHT OF WAY Coordinator.  In the event that changes or alterations are approved, a revised centerLINE plat with the new locations shall be recorded BY THE DESIGN CONSULTANT’S SURVEYOR in THE county recorder’s office.